SolanaWatch: Oral Defense Evaluation Compliance Matrix

SolanaWatch: Capstone Proposal Defense Revision Matrix & Blueprint

Project Title: SolanaWatch: An IoT-Based Disease Early Warning System for Soil-Grown Solanaceae Crops

Institution: University of Santo Tomas – College of Information and Computing Sciences (Department of Information Technology)

Academic Year: Special Term, A/Y 2025–2026

Adviser: Prof. Eugenia R. Zhuo, DIT

Evaluation Panel: Prof. Noel E. Estrella, Mr. Karlo Angelo Tino, Inst. Gabriel Emanuel D. Montano (Head Panel)

1. Executive Summary & Defense Evaluation Overview

The oral defense evaluation summary returned a verdict of Major Revisions. The primary areas identified by the panel for enhancement include:

Hardware Engineering Rigor & Power Calculations: Transitioning from an unverified battery setup (3.7V 2600 mAh) to a mathematically sized and empirically validated Solar-Assisted Power Subsystem (12V LiFePO₄ + MPPT + 20W PV panel), with complete power audits and battery autonomy calculations.

Scientific & Pathological Basis: Establishing explicit mathematical models and literature-grounded rules (Wallin’s Severity Values / BLITECAST adaptation for Late Blight; soil thermal-moisture index for Bacterial Wilt) rather than arbitrary thresholds.

Architectural & Hardware Detail: Providing exact sensor models, RS485 Modbus communication interfaces, custom PCB justifications over breadboards, Stevenson radiation shield housing, and IP65/IP66 enclosure specifications.

System Reliability, Synchronization & Edge Fail-Safe: Formalizing the SPI MicroSD FIFO offline data logging buffer, idempotent HTTP data synchronization with server ACK, de-duplication logic, and comprehensive sensor calibration procedures.

Quality Assurance & Verification Matrix: Adding granular ISO/IEC 25010 aligned test cases for component unit testing, wireless RSSI communication range decay, battery autonomy, solar charging profiles, and field validation.

2. Complete 24-Point Panel Revision Action Matrix

| # | Panel Comment / Revision Item | Target Section | Identified Weakness in Initial Draft | Required Academic & Technical Action |
| --- | --- | --- | --- | --- |
| 1 | Add comparative analysis of ESP32, Arduino Leonardo, and Raspberry Pi in Chapter 2. | Chapter 2 (RRL) | Qualitative discussion lacked structured tabular metrics and micro-architecture comparison. | Add a comparative table comparing Clock Speed, SRAM, Flash, Built-in Wireless, Deep-Sleep Current, Active Power Draw, Unit Cost, and Modbus/SPI/I2C interfacing. |
| 2 | Justify selection of ESP32 based on SolanaWatch's requirements. | Chapter 2 & Chapter 3.1 | Justification was generic without mapping to edge logging, RS485 communication, and solar autonomy. | Explicitly tie ESP32 dual-core Xtensa LX6, native 2.4 GHz Wi-Fi stack, low-power deep sleep (10–150 µA), and peripheral buses (SPI for SD, UART2 for RS485, ADC for capacitive sensor) to system requirements. |
| 3 | Specify exact models and specifications of all sensors and components. | Chapter 3 (System Req. - Table 6) | Generic sensor descriptions without part numbers, tolerances, operating voltages, or signal protocols. | List exact part numbers: DHT22/AM2302 (Air Temp & RH), DS18B20 (Waterproof Stainless Probe, 1-Wire), Capacitive Soil Moisture Sensor v1.2 (Analog), IP68 Industrial RS485 Modbus Leaf Wetness Sensor, MAX485/SP3485 TTL-RS485 transceiver. |
| 4 | Reassess the 3.7 V 2600 mAh Li-ion battery for 24/7 operation. | Chapter 2 & Chapter 3.1 | 3.7V 2600 mAh (9.62 Wh) depletes in < 16 hours with active Wi-Fi and 12V RS485 boost converters. | Perform a formal power discharge audit proving why single 18650 Li-ion cells fail 24/7 outdoor operation and justify upgrading to a 12.8V LiFePO₄ battery chemistry. |
| 5 | Investigate the use of a solar-assisted power system for continuous operation. | Chapter 2 & Chapter 3.1 | Solar power was mentioned without engineering sizing formulas or solar irradiance analysis. | Provide solar photovoltaic sizing models based on Philippine Peak Sun Hours (PSH = 4.5 h/day), solar charging efficiency (η = 0.75), and continuous day/night energy cycling. |
| 6 | Conduct actual power-consumption testing of the complete prototype. | Chapter 3.4 (Test Plan) | Missing physical power measurement protocol and energy dissipation profiles. | Design a power measurement test bench using a digital multimeter/shunt resistor to capture current draw across Active TX (180–240 mA), Sensing (30–60 mA), and Deep Sleep (15–50 µA) states. |
| 7 | Determine appropriate solar panel and battery capacity based on measured power requirements. | Chapter 3.1 & 3.3 | Hardware ratings were selected arbitrarily without mathematical sizing formulas. | Integrate formal sizing formulas: Daily Energy Budget (E_daily), Battery Capacity (C_battery with 3 Days of Autonomy and 80% DoD), and PV Panel Wattage (P_solar). |
| 8 | Clarify the difference between sensor coverage and wireless communication range. | Chapter 1 (Scope) & Chapter 3.1 | Conflated microclimatic sensing footprint with ESP32 Wi-Fi transmission radius. | Differentiate Microclimatic Representative Footprint (5–15 m radial zone of microclimate homogeneity) vs. RF Wireless Range (30–50 m Wi-Fi 802.11 b/g/n line-of-sight). |
| 9 | Conduct experimental test to determine actual communication range of the ESP32. | Chapter 3.4 (Test Plan) | Lacked empirical RF range and signal degradation testing protocol. | Formulate an experimental RSSI (dBm) and Packet Delivery Ratio (PDR %) test at 5m, 10m, 20m, 30m, 40m, and 50m intervals under Line-of-Sight (LOS) and crop canopy Non-Line-of-Sight (NLOS). |
| 10 | Provide RRL support for selected environmental parameters. | Chapter 2 (RRL) | Need stronger plant pathology citations linking microclimate to specific Solanaceae pathogens. | Provide direct peer-reviewed citations for Air Temperature, RH, Leaf Wetness Duration (LWD), Soil Temperature, and Soil Moisture influencing Phytophthora infestans and Ralstonia pseudosolanacearum. |
| 11 | Provide research basis for all disease-risk thresholds and formulas. | Chapter 3.1 (Table 11) | Thresholds were presented as static ranges without mathematical combination logic or disease model grounding. | Formulate Wallin's Severity Values (SV) / BLITECAST rule-base for Late Blight and hydrothermal soil infection indices for Bacterial Wilt with exact scientific citations. |
| 12 | Clarify how multiple environmental parameters are combined to determine disease risk. | Chapter 3.1 & 3.3 | No multi-variate formula explaining how individual sensor thresholds synthesize into a single risk index. | Define decision tree algorithms combining air, soil, and canopy metrics over cumulative time windows (LWD ≥ 10 h). |
| 13 | Clarify that SolanaWatch performs disease-risk assessment rather than direct disease diagnosis. | Chapter 1 (Scope & Delimitations) | Ambiguous terminology implied that the system detects pathogen existence directly. | Explicitly refine all text to state that SolanaWatch evaluates microclimatic conduciveness (risk modeling) and does not perform biological/optical pathogen detection. |
| 14 | Identify intended users of SolanaWatch. | Chapter 1 (Purpose) & Chapter 3.1 | User descriptions were broad without persona and operational context. | Define specific target user groups: Smallholder Solanaceae farmers, agricultural technicians, screenhouse operators, and agricultural researchers. |
| 15 | Clearly label all components and functions in circuit and system diagrams. | Chapter 3.3 (System Design) | Circuit schematic lacked GPIO pin mappings, pull-up resistors, power rail lines, and level shifters. | Produce an annotated schematic showing ESP32 pinouts (GPIO 4: OneWire, GPIO 16/17: UART2 Modbus, GPIO 34: ADC, GPIO 5/18/19/23: SPI SD), step-down buck converters, and 4.7kΩ pull-up resistors. |
| 16 | Evaluate using a PCB instead of a breadboard for the final prototype. | Chapter 3.1 & 3.3 | Breadboards are vulnerable to corrosion, contact bounce, and outdoor field failure. | Add a comparative evaluation proving that a custom PCB / Proto-Shield ensures low contact resistance, resistance to tropical humidity, anti-vibration integrity, and clean signal paths. |
| 17 | Add test cases for sensor accuracy, data transmission, offline logging, and data synchronization. | Chapter 3.4 (Test Plan) | Existing test cases were vague ("Test sensor reads climate"). | Implement structured test suites with Test Case ID, Description, Preconditions, Test Steps, Expected Result, and Acceptance Criteria. |
| 18 | Explain how data loss and duplication will be prevented during network interruptions. | Chapter 3.3 & 3.4 | Sync mechanism lacked technical description of deduplication and ACK handshaking. | Detail the SPI-linked MicroSD FIFO buffer, monotonic timestamping, transactional batch sync, HTTP 200/201 ACK validation, and database UNIQUE(node_id, recorded_at) constraint. |
| 19 | Establish calibration procedure for all environmental sensors. | Chapter 3.4 (Test Plan) | Missing sensor calibration methods prior to field deployment. | Provide standard 2-point calibration for capacitive soil moisture (air vs. water), ice-point/boiling calibration for DS18B20, and saturated salt chamber verification for DHT22. |
| 20 | Specify Solanaceae crops and field conditions covered by the study. | Chapter 1 (Scope & Delimitations) | Broad reference to Solanaceae without crop varieties and agro-ecological conditions. | Specify Tomato (Solanum lycopersicum), Eggplant (Solanum melongena), Bell Pepper (Capsicum annuum), and Potato (Solanum tuberosum) in open-field and screenhouse cultivation under Philippine climatic conditions. |
| 21 | Evaluate enclosure and components for outdoor agricultural conditions. | Chapter 3.1 & 3.3 | Enclosure lacked ingress protection ratings, UV considerations, and thermal radiation shielding. | Specify IP65/IP66 weatherproof junction box with silicone gaskets, PG7/PG9 waterproof cable glands, silica desiccant packets, and a louvered Stevenson radiation shield for the DHT22. |
| 22 | Add reliability testing plan for continuous operation. | Chapter 3.4 (Test Plan) | Lacked continuous operational stress testing and watchdog timer fail-safe protocols. | Add a 72-hour and 7-day continuous unattended field testing protocol, ESP32 hardware Watchdog Timer (WDT) configuration, brown-out reset (BOR), and auto-recovery routines. |
| 23 | Provide validation procedure for solar charging and battery autonomy. | Chapter 3.4 (Test Plan) | No procedure to verify if battery survives zero-sunlight conditions or charges efficiently. | Detail a 3-day solar deprivation autonomy stress test (battery discharge curve) and a 48-hour solar harvesting profile under varying ambient solar irradiance (W/m²). |
| 24 | Support all technical claims and component selections with recent RRL, datasheets, calculations, or experimental results. | Entire Manuscript (Chapters 1–3) | Several claims lacked direct citations or empirical justification. | Integrate 2020–2026 high-impact journal citations (IEEE, Elsevier, Springer, MDPI), official datasheets, and derived engineering formulas across all chapters. |