|  | UNIVERSITY OF SANTO TOMAS COLLEGE OF INFORMATION AND COMPUTING SCIENCES DEPARTMENT OF INFORMATION TECHNOLOGY |  |
| --- | --- | --- |

REVISION MATRIX

| PROGRAM: | Bachelor of Science in Information Technology |
| --- | --- |
| DEFENSE DATE: | July 20, 2026 |
| TITLE: | SolanaWatch: An IoT-Based Disease Early Warning System for Soil-Grown Solanaceae Crops |
| MEMBERS: | 1. Morales, Simeon Agabus S. 2. Rapada, Raven M. 3. Hernandez, Andrei S. 4. Eugenio, Vince Angelo |
| ADVISER: | PROF. EUGENIA R. ZHUO, DIT |

| COMMENTS / RECOMMENDATIONS | ACTION TAKEN | PAGE REFLECTED |
| --- | --- | --- |
| Add a comparative analysis of ESP32, Arduino Leonardo, and Raspberry Pi in Chapter 2. | Added a comparison of ESP32, Arduino Leonardo, and Raspberry Pi 4 in Chapter 2, evaluating their processor speed, memory, power consumption, and outdoor field suitability. | Page 21, Page 22, Pages 88–90 |
| Justify the selection of the ESP32 based on SolanaWatch's requirements. | Explained why ESP32 was chosen based on its dual-core processor, built-in Wi-Fi, low-power deep sleep mode, low cost, and support for multiple sensor pins. | Page 22, Page 32, Page 41, Page 57 |
| Specify the exact models and specifications of all sensors and components. | Updated the hardware list and Chapter 2 with exact part models: DHT22 (air temp/RH), DS18B20 (soil temp), Capacitive Soil Moisture v1.2, RS485 Leaf Wetness sensor, MicroSD module, 20W solar panel, MPPT controller, and 12.8V LiFePO4 battery. | Pages 22–26, Pages 41–49 |
| Reassess the 3.7 V 2600 mAh Li-ion battery for 24/7 operation. | Replaced the 3.7V 2600 mAh battery with a 12.8V 12Ah LiFePO4 battery pack after a power calculation showed the single cell cannot support 24/7 continuous operation. | Pages 25–26, Page 44, Page 68 |
| Investigate the use of a solar-assisted power system for continuous operation. | Added a solar-assisted power system using a 20W solar panel and an MPPT charge controller to allow continuous 24/7 field operation without grid power. | Pages 25–26, Page 44, Pages 57–58, Page 68 |
| Conduct actual power-consumption testing of the complete prototype. | Included power measurement test cases in the test plan to test the current draw of the ESP32 during active Wi-Fi sending, sensor reading, and deep sleep. | Page 26, Page 68, Pages 78–80 |
| Determine the appropriate solar panel and battery capacity based on the measured power requirement. | Calculated the solar panel size (20W) and battery size (12.8V 12Ah / 153.6 Wh) using Philippine sunlight hours and the system's estimated daily power use of 2.30 Wh/day. | Page 26, Page 68 |
| Clarify the difference between sensor coverage and wireless communication range. | Clarified that sensor coverage is the localized area around the sensors (5 to 15 meters), while wireless range is the Wi-Fi distance between the ESP32 and the router (30 to 50 meters). | Pages 27–28 |
| Conduct an experimental test to determine the actual communication range of the ESP32. | Added a test case in Chapter 3 to test Wi-Fi signal strength (RSSI) and packet delivery at 5m, 10m, 20m, 30m, and 50m distances. | Page 28, Page 79, Page 81 |
| Provide RRL support for the selected environmental parameters. | Added research citations showing how air temperature, humidity, leaf wetness, soil temperature, and soil moisture affect Late Blight and Bacterial Wilt. | Pages 5–6, Pages 18–21, Pages 23–24, Pages 31–32 |
| Provide the research basis for all disease-risk thresholds and formulas. | Added research references for the disease thresholds, using Wallin's Severity Values for Late Blight and a hydrothermal soil formula for Bacterial Wilt. | Pages 19–20, Pages 30–31, Pages 61–63, Page 65 |
| Clarify how multiple environmental parameters are combined to determine disease risk. | Explained how sensor readings are combined into decision rules to classify disease risk into Low, Moderate, and High alert levels. | Pages 30–31, Page 57, Pages 61–63, Page 65, Page 72 |
| Clarify that SolanaWatch performs disease-risk assessment rather than direct disease diagnosis. | Clarified throughout the paper that SolanaWatch assesses environmental disease risk and favorable weather conditions, rather than diagnosing actual plant infection. | Page 8, Page 14, Page 15, Pages 30–31 |
| Identify the intended users of SolanaWatch. | Identified smallholder farmers as primary users, and agricultural technicians, extension workers, and researchers as secondary users. | Pages 12–14, Page 51, Pages 85–87 |
| Clearly label all components and functions in the circuit and system diagrams. | Updated and labeled all system diagrams and circuit schematics with component names, GPIO pin numbers, and power connections. | Page 13, Page 58, Page 64, Page 66, Page 71, Page 74 |
| Evaluate using a PCB instead of a breadboard for the final prototype. | Added an explanation for using a custom PCB instead of a breadboard to ensure durable, secure connections and prevent loose wires outdoors. | Page 28, Page 32, Page 49 |
| Add test cases for sensor accuracy, data transmission, offline logging, and data synchronization. | Added a complete test cases table in Chapter 3 covering sensor accuracy, Wi-Fi sending, offline SD card logging, and cloud data sync. | Pages 76–82 |
| Explain how data loss and duplication will be prevented during network interruptions. | Explained that data is saved to a MicroSD card during network loss, uploaded in batches upon reconnecting, and deduplicated in the database using unique timestamps. | Pages 26–27, Page 39, Pages 58–59, Pages 66–67 |
| Establish a calibration procedure for all environmental sensors. | Added calibration steps for sensors, including dry/wet calibration for soil moisture, ice-water testing for soil temperature, and salt chamber testing for humidity. | Pages 24–25, Pages 76–77 |
| Specify the Solanaceae crops and field conditions covered by the study. | Specified four Solanaceae crops (tomato, eggplant, pepper, and potato) under open-field and screenhouse soil conditions in the Philippines. | Pages 5–6, Page 15, Pages 17–19 |
| Evaluate the enclosure and components for outdoor agricultural conditions. | Selected an IP65/IP66 weatherproof enclosure box with rubber gaskets, cable glands, desiccant packs, and a Stevenson shield for outdoor protection. | Page 28, Page 42, Page 45, Page 70, Page 74 |
| Add a reliability testing plan for continuous operation. | Added a 7-day continuous field test plan, hardware watchdog timer auto-reset, and water spray testing to ensure system reliability. | Page 39, Pages 77–78, Pages 81–82 |
| Provide a validation procedure for solar charging and battery autonomy. | Added a 3-day (72-hour) battery test without sunlight and a solar charging test under daylight to verify battery life and charging performance. | Page 68, Page 77, Page 80 |
| Support all technical claims and component selections with recent RRL, datasheets, calculations, or experimental results. | Supported all hardware choices, formulas, and technical details with 2020–2026 research papers, official datasheets, and calculations in APA 7th format. | Pages 5–33, Pages 41–50, Pages 57–68, Pages 93–101 |

| PANEL MEMBERS |  |  |
| --- | --- | --- |
| INST. GABRIEL EMANUEL D. MONTANO | PROF. NOEL E. ESTRELLA | MR. KARLO ANGELO TINO |
| NAME AND SIGN OF LEAD PANEL | NAME AND SIGN OF MEMBER PANEL 1 | NAME AND SIGN OF MEMBER PANEL 2 |